OPEN OVERLAND

Door Controller Module

Hardware Specification v1.3

Confidential — Open Overland OS Project

Overview

Standalone PCB controlling 2 electric door latches via NFC fob authentication. One module per 2 doors. Mounts anywhere inside aluminum storage box. Keep in mind that it could block Wi-fi/Bluetooth signal

Apple HomeKit compatible via HomeSpan firmware. ESP32 allows HomeSpan firmware, but its development is not included in this job.

1.  Power Input

- 12V DC from vehicle battery

- XT30 connector (rated 30A) - I could do that, but previously I suggested to use the same Molex MicroFit 3.0 (there is no reason for high currents like 15 Amps for such a small board)

- Onboard automotive blade fuse (10A) - Blade Fuse acceptable, but I’d suggest to use 2-5A

- Onboard 3.3V LDO regulation for MCU and NFC

- Reverse polarity protection — AO3401 P-channel MOSFET

- TVS diode on 12V rail — SMBJ13A (load dump spike protection)

2.  Compute Module

- ESP32-S3-WROOM-1 soldered directly on board

- WiFi 802.11 b/g/n + Bluetooth 5.0 LE included

- USB-C port for programming and OTA firmware updates - OTA firmware updates depends on initial firmware developed

- Reset button

- Boot button (firmware flash mode) – not really necessary since USB used for programming

- 1× status LED green (power / armed)

- 1× status LED red (alarm / unlocked)

3.  NFC Reader

- 1× PN532 chip on board — I2C to ESP32-S3

- Onboard antenna DISABLED

- External antenna via U.FL connector on PCB – this is kind of unusual connector for NFC (13.56Mhz vs. 2.4GHz) which could malfunctioning: the best solution is to get off-the-shelf NFC-module or to place PN532 chip very close to the antenna (i.e. on a separate board apart from ESP32). Otherwise you should go for coaxial cables and I should offer track-tuning to fit 50 Ohm Z0 impedance with the further debugging.

- U.FL to SMA coax pigtail cable — 30cm length – not sure that it is good idea, These connectors designed for  2.4GHz

- External antenna: flat PCB patch antenna, 13.56MHz tuned, ~40mm × 40mm, SMA connector – I’d recommend to place PN532 chip on that board. Communication between this board and main board around ESP32 will be much easier with the regular 4 wires (I2C-interface + power).

- Antenna mounts behind door panel — completely invisible from outside

- Reads ISO 14443A NFC fobs and cards

- Reads through 3mm aluminum at 3–5cm range – It will not work! NFC signal could not pass Aluminium

- Antenna positioned by installer at any door panel location

4.  Latch Outputs (2×)

- 2× Molex Microfit 3.0 female 6-pin connector — one per door latch

- Switching: 2N2222A NPN transistor per latch – Possible, but I’d recommend High-side switch by P-MOSFET

- Maximum current per latch: 1A continuous

- Decoupling: 100nF capacitor per latch circuit – 1 or even 10 uF is better

- 10kΩ pull-up resistors on Pin 4 (LATCH STATUS) and Pin 6 (DOOR STATUS)

- FAIL-SECURE: GPIO LOW or floating = latch LOCKED — circuit level guarantee

Connector pinout per latch port:

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | +12V | Always on — from vehicle battery |
| 3 | CTRL | Switched 12V — transistor output |
| 4 | LATCH STATUS | Digital input — 10kΩ pull-up to 3.3V |
| 5 | NC | Not connected |
| 6 | DOOR STATUS | Digital input — 10kΩ pull-up to 3.3V |

5.  Light Output (1×)

- Switching component: IRLZ44N N-channel MOSFET (logic level, 3.3V gate drive) - Possible, but I’d recommend High-side switch by P-MOSFET

- Minimum 5A continuous at 12V — 60W capacity

- Heatsink pad on PCB under MOSFET

- Output connector: 2-pin screw terminal (12V switched + GND) – I assume that screw terminals are not vibration-proof and rather recommend a spring-loaded clamps.

- For: 12V LED strip lighting inside aluminum storage box

- Auto-triggered by door open event (software controlled)

6.  Tamper Input (1×)

- 1× digital input — 2-pin JST connector – OK, do you have any preferences? They are available with the pitch from 1.25 to 3.75 mm. Most popular are 2.00 and 2.50 mm.

- 3.3V logic level

- 10kΩ pull-up resistor on board

- Normally closed circuit

- Circuit opens = tamper event = alarm triggered in software

- For: hidden switch detecting forced box entry

7.  I2C Expansion Port (1×)

- 4-pin JST connector: 3.3V — GND — SDA — SCL

- Plug any I2C sensor without board redesign

- Examples: light sensor (auto lights), temperature sensor, ADS1115 ADC (any analog sensor), microphone level module

8.  Board Requirements

- Compact form factor — fits inside aluminum storage box

- 4× M3 mounting holes (3.2mm inner diameter)

- All connectors on same board edge for clean cable routing

- 2-layer PCB minimum

- Conformal coat recommended for moisture protection

- Operating temperature: -20°C to 80°C (Baja California rated)

9.  Fail-Secure Requirement (Critical)

The following conditions must ALL result in latches LOCKED:

- Power loss → latches LOCKED

- GPIO floating → latches LOCKED

- MCU crash → latches LOCKED

- Firmware boot sequence → latches LOCKED

- Any undefined state → latches LOCKED

This is a hard circuit requirement. Transistor must be OFF by default. Must not depend on software state.

10.  Compatible Latch

| Specification | Value |
| --- | --- |
| Part Number | Southco R4-EM-9P15-150 |
| Trigger Spring | Detent Pull to Open |
| Relock Type | Delayed |
| Microswitch | Door + Latch Status |
| Connector | Non-sealed |
| Mounting | 1/4-20 thread |
| Housing | Aluminum die cast |
| Voltage | 12–24V DC |
| Pry Resistance | 5,800N (1,304 lbs) |
| Electronic Release | 900N (202 lbs) |
| Cycle Life | 90,000 cycles |

11.  Connector Summary

| Direction | Connector | Function |
| --- | --- | --- |
| IN | XT30 2-pin I’d recommend to use Molex Microfit 2-pin | 12V power from vehicle battery |
| IN | USB-C | Programming + firmware updates |
| OUT | Molex Microfit 6-pin × 2 | Latch control — one per door |
| OUT | Screw terminal 2-pin – I’d recommend to use Spring-loaded clamp terminals | Light output — 5A, 12V switched |
| IN | JST 2-pin | Tamper switch input |
| I/O | JST 4-pin | I2C expansion port |
| RF | U.FL – I don’t recommend | External NFC antenna coax |

12.  Component BOM Summary

| Component | Part | Function |
| --- | --- | --- |
| Compute module | ESP32-S3-WROOM-1 | MCU — WiFi + BLE |
| NFC controller | PN532 | NFC reader chip |
| Light MOSFET | IRLZ44N | Light output 5A |
| Latch transistors | 2N2222A × 2 | Latch switching |
| Reverse polarity | AO3401 | P-MOSFET protection |
| Spike protection | SMBJ13A | TVS diode 12V rail |
| Overcurrent | 10A blade fuse | Input protection |
| Decoupling | 100nF × 2 | Latch circuits |
| Pull-ups | 10kΩ × 4 | Status pin pull-ups |
| Latch connectors | Molex 43025-0600 × 2 | 6-pin Microfit 3.0 |
| Antenna port | U.FL connector | External NFC antenna |
| Status LEDs | Green + Red | Power + alarm |
| Buttons | Reset + Boot | Firmware control |

13.  Software — Not Anton's Scope

- HomeSpan library → Apple HomeKit integration

- Each latch → Lock accessory in Apple Home app

- Light output → Light accessory in Apple Home app

- Tamper input → Security sensor in Apple Home app

- I2C sensor → Temperature / Light sensor in Apple Home app

- NFC authentication → fob whitelist management

- WiFi modes → AP (hotspot) + Station (connect to network) simultaneously

- OTA firmware updates → over WiFi

- License → MIT open source

- Repository → github.com/openoverland

Open Overland OS — The open standard for truck camper builds.

Engineering-grade specs and vetted components — open source and free.